ADDENDUM #1

LIFT STATION NO. 6 PUMP REPLACEMENT

CITY OF DULUTH

PROJECT 0743SN

BID NUMBER 08-0664

BID OPENING DATE - DECEMBER 17, 2008

NOTICE 									Page 1 of 1

This Addendum is issued to modify, explain or correct the original drawings, specifications and/or previous addendums and is hereby made a part of the Contract Documents. Please attach this Addendum to the specifications in your possession and note receipt of this Addendum on page 2 of the request for bid. The bid date remains unchanged.

PROJECT MANUAL

SPECIAL PROVISIONS

ADD the following statement to SP-1 SCOPE OF WORK:

Contractor shall replace existing isolation gate valve on the suction side of the pump with a new gate valve provided by the City.

ADD the following paragraphs to SP-9 MATERIALS AND WORK TO BE FURNISHED BY THE PUBLIC WORKS & UTILITIES DEPARTMENT:

SP-9.2 Contractor is responsible for setting, shimming, and aligning the pump, providing startup services, and perform hydraulic and performance testing.

SP-9.3 The City will provide the isolation gate valve to be installed on the pumps suction

piping. The valve is currently located in the Lift Station No. 6 Dry Well.

ADD the following paragraphs to SP-10 EXISTING LIFT STATION NO. 6 FACILITIES:

SP-10.4 Contractor shall be responsible to monitor atmospheric conditions while working in the Lift Station Dry Well.

SP-10.5 Contractor shall remove and dispose of existing pump, motor, and base. Contractor shall salvage mechanical seal assemblies from the pump and deliver to the City.

CONTRACT DRAWINGS

ADD General Note #13 to Sheet 1 of 2:

13. Contractor may use existing column adjacent to Pump 1 as a support for vertical

electrical conduit. Coordinate with Engineer as location of conduit runs can vary.

DELETE AND REPLACE the plan note on Sheet 1 of 2 reading “EXISTING GATE VALVE TO REMAIN” with “INSTALL NEW GATE VALVE FURNISHED BY OTHERS”.

END OF ADDENDUM