CITY OF DULUTH

MINIMUM SPECIFICATIONS FOR

ARTICULATING 6-WHEEL ALL WHEEL DRIVE MOTOR GRADER

LEASE/PURCHASE 16-20 UNITS

Contacts: Technical questions-Courtney Petersen-218-730-5430

About the bid-Dennis Sears-218-730-5003

All bids are to meet or exceed specifications listed and described as follows:

General

It is the intent of these specifications to describe an all-wheel drive articulated diesel powered motor grader in sufficient detail to secure bids on comparable equipment.  All parts not specifically mentioned, which are necessary in order to provide a complete grader shall be included in the bid and shall conform in strength, quality of material and workmanship to what is usually provided the trade in general.

Any units not conforming to these specifications as outlined in this invitation will be rejected and it will be the responsibility of the bidder to conform to the requirements unless deviations have been cited in the bid and acceptance made on that basis.

Any additional, deletions or variations from the following specifications must be stated.  Reason for variations and deviations must also be stated.  These specifications shall be construed as minimum.  The manufacturer shall furnish evidence upon request of its demonstrated ability to build equipment of the type described in these specifications.  Evidence shall be furnished upon request that the equipment built by the manufacturer has been tested and proven in the market place.  Specifications also require that the bidder furnish descriptive literature, complete specifications, and all other necessary data on the equipment proposed to furnish.  (Use attachment, if necessary)  Furnish copy of manufacturer’s warranty.

Location of parts outlet may be used in the evaluation of bid proposals.

Failure to complete all information on questionnaire and bid request form may be cause for rejection of bid.

Parts and Services

Because maintenance of this equipment in good operational condition without protracted time out for repair is very important, it is essential that repair parts and service be adequate and readily available.  Response time for necessary repairs must be less than one (1) hour; seven (7) days per week, 24 hours per day.  State location of bidder’s service and repair parts facility.

Furnish name, address and telephone number of the nearest parts and service outlet/depot.  Attach list of additional parts outlets/depots, if any.

For Parts Contact:				For Service Contact:

Name: 		_____________________________		________________________________

Address:	_____________________________		________________________________

Telephone:	_____________________________		________________________________

Delivery

Motor graders are to be bid F.O.B. Destination, Fleet Service Division, 4825 Mike Colalillo Drive, Duluth, Minnesota.  Each unit is to be delivered in first class operating conditions with acceptance subject to purchaser’s inspection and approval.

Basic cost of one new or used grader will be part of determining costs of the grader award but not the sole determining facors.  Please state a quantity discount of new or used graders as a %. All options will be added after the award is made and as determined by the end user.

Manufacturer:	________________________________________________________________________

Model:		________________________________________________________________________

Description			Specification					Bid Response

Type				All-wheel drive, articulated frame		___________________

Operating Weight		33000 lbs. SAE minimum (without scarifier,	___________________

add-on weights or wheel ballast)

Engine				6 cylinder turbo charged diesel must be EPA	___________________

Tier II certified

Rated not more than 2100 RPM and not less

than 436 cu. in. displacement with minimum

of 50% Torque rise

Variable HP w/ min 185 Net HP			___________________

In all wheel drive				___________________

HP in gears 1 and 2				___________________

HP in gears 3 – 5				___________________

HP in gears 6 – 7 				___________________

HP in gear 8					___________________

Electrical System		24 to 12 volt converter for accessories		___________________

Heavy duty 75 AMP alternator			___________________

Batteries rated at 1300 CCA			___________________

Cooling System			Anti-freeze solution to -40⁰ F, extended life	___________________

Transmission			Direct Drive, Electronic controlled with auto	___________________

shift, transmission reversal without clutch

and over speed protection.

5 working speeds below 10 MPH		___________________

Max transport speed shall be no less than 	___________________

25 MPH

Min reverse speed shall not be less than 	___________________

20 MPH

Hydraulic System		Maximum pressure not less than 2,750		___________________

standby PSI, pressure compensated load

sensing system with 54 GPM flow rating

Integrated wing controls on rack		___________________

Tandem Drive Axles		No spin, foot or electric operated 		___________________

hydraulically actuated rear axle differential

lock and unlock able on-the-go under full

engine RPM in any gear

Front Wheel Drive (FWD)	Front wheel drive engageable on the go in 	___________________

any direction and under full engine RPM

and load.  Front wheel drive on demand,

engage able from a dead stop

Manual or auto torque adjustable FWD 		___________________

operated from inside the cab for operator

management.  FWD shall be effective from

0 to 13 MPH in reverse gears.

Steering			Full hydraulic power steering with separate	___________________

reservoir or priority hydraulics for safety

in the event of hose failure

Maximum turn radius shall be no greater	___________________

Than 24’ 6”

Brakes, Parking, Service		Air or Hydraulic actuated, four wheel oil		___________________

disc brakes for (air dryer required, if applicable)

Park brake to be independent of service 	___________________

brake, automatically or manually actuated

transmission neutralizing for safety on hills

Braking system must meet SAE J/ISO 3450	___________________

standards

Moldboard and impact 		Circle drive must have a slip clutch		___________________

Protection

Blade lift cylinders must have crossover		___________________

relief valve or approved blade impact system

Moldboard			Fully hydraulic controlled moldboard 12 ft. 	___________________

long, 24 inches tall, 1 nominal inch thick with

8 inch cutting edge

Fully hydraulic controlled moldboard 14 ft.	___________________

long, 24 inches tall, 1 nominal inch thick with

8 inch cutting edge

Lights				LED turn, brake and warning Strobe light	___________________

with guard (blue)

Front bar 	2 drive, low beam

2 drive, high beam

Mid Frame	2 work circle

Under Cab	2 work blade

Rear grill	2 work backup

Front roof	2 drive (inner)

2 work (outer)

Rear roof	2 work rear

RH side roof	2 work RH side

Work lights to be switched separately		___________________

Cab				Low profile, sit down cab			___________________

Interior sound level 75 dba or less		___________________

Front (upper & lower) & rear windshield 	___________________

wipers and washers

Rear window defrost				___________________

Outside heated mirrors (2)			___________________

Air-suspension cloth seat with lumbar 		___________________

support, arm rests and headrest

Tinted safety glass ½ deep tint			___________________

AM/FM radio with weather band		___________________

Heavy-duty 40,000 BTU heaters & defrost   	___________________

for winter use

Air-conditioned					___________________

Gauges & Monitoring		All gauges to be lighted				___________________

Hour meter					___________________

Electronic control module w/ data storage	___________________

and diagnostic capabilities for engine,

transmission and 6 wheel drive systems

Three level electronic warning system for all	___________________

critical machine functions

Fuel System			Minimum of 100 Gallons			___________________

Safety				Slow moving vehicle sign vandalism 		___________________

protection (lockable doors & compartments),

back-up alarm and tool box

Tires				14:00 x R24, Michelin x-snow plus or 		___________________

Bridgestone v-steel snow wedge

Used machines should have a minimum of 50%

tread at the start of each winter season

2 front spare tires and 2 rear spare tires		___________________

Special Equipment

Snow Wing		Subject to approval by Fleet Services		___________________

List manufacturer				___________________

All hydraulic 12 ft snow wing to include heel,	___________________

toe and rear post

Integrated controls to be mounted on left	___________________

of the left rack, wing to be punched AASHO

standard

One stage cylinder on the wing			___________________

Minimum 12” Float detent on the wing		___________________

Front Attachment	Hydraulic parallel lift quick attach/detach	___________________

front lift group to pick up existing V-plows

and dozer blades

Manuals			Each unit to include owner/operator manual	___________________

One parts manual & one service manual in 	___________________

book form or CD format

Training			Vendor/manufacturer agrees to provide a 	___________________

Training program for the City of Duluth

Maintenance & Operator employees in

sufficient scope to assure efficient and

economical performance of the equipment.

Training to commence at a reasonable time

(shortly after acceptance) agreeable to both

parties.

Optional Equipment:		List cost in addition to proposed monthly lease,

per unit, per month.

Front fenders

List Manufacturer			___________________

Lease: 5 year/monthly rate		___________________

Joystick for Motor Grader

Maddock Industries/Mfgr

Lease: 5 year/monthly rate		___________________

Dozer Blade, 9ft & must couple to City’s

existing attachments

List Manufacturer			___________________

Lease: 5 year/monthly rate		___________________

Hydraulic stiff arm on wing

List Manufacturer			___________________

Lease: 5 year/monthly rate		___________________

The City of Duluth reserves the right to accept or reject any options included in bid.

Bidders shall list deviations from bid specifications (additions or deviations of requested equipment).  All questions shall be directed to Arlen Bordenkircher at abordenkircher@duluthmn.gov or John McIntyre at jmcintyre@duluthmn.gov .  Questions must be communicated no later than seven (7) days prior to the bid opening date in order to facilitate the response for all potential bidders.

All rates bid shall be for a total of 2500 hours over the entire length of the five winter season lease.

The City of Duluth reserves the right to cancel any agreements resulting from this bid for non-performance of equipment or failure of the City Council to approve sufficient appropriations for the portions of the rental period beyond 2010.

Note: Used motor graders should not have any more than 1500 hours or be more then 5 years old (from the date of manufacture) at start of the 2010/2011 winter season.  Used motor graders should have been used in a similar setting/manner (i.e. city, county, township). All units are subject to city inspection and approval.

The City of Duluth will be responsible for daily operator checks and inspections of graders (prior use).  Routine maintenance and check points at ground level for safety purposes.

The successful vendor will supply any required maintenance and repairs at no cost to the City, including tire repairs, and all scheduled maintenance as per the manufacturers’ recommendations.  The vendor will not be responsible for any damage done by City use which is beyond what is considered normal wear and tear.

However, if more than 6 moldboards are broken during a season, the vendor will bear responsibility for replacements.  The replacements must be within the time frame for repairs.

The City of Duluth is responsible for cutting edges and applicable nuts.

The successful vendor shall respond to all ‘unit down time’ service requests within 1 hour of notification.  The City of Duluth reserves the right to terminate the lease agreement if vendor does not comply with service requirements.

If a leased unit is down for repairs in excess of 48 continuous hours from notification (including nights, weekends and holidays), a like 6 wheel drive unit with wing is required as a replacement (at no additional cost to the City of Duluth).  The vendor will not be required to provide a replacement unit for any ‘down time’ resulting from damage done by City use that is beyond what is considered normal wear and tear.

The City of Duluth has the right to charge liquidating damages in the amount of 10% of the monthly lease/rental cost if supplied grader differs from above agreement.

At the time of the bid opening, each vendor will be presumed to have read and to be thoroughly familiar with the bid documents, including any and all addendums, and to understand the City’s needs with regard to snow removal and the proper equipment required to complete such removal.

Vendor Contact Name/Phone Numbers

Repair Contact Name:		______________________________	Phone:	_____________	______

Cell:	___________________

Sales Person Name:		______________________________	Phone:	___________________

Cell:	___________________

Local Manager Name:		______________________________	Phone:	___________________

Cell:	___________________

District Manager Name:		______________________________	Phone:	___________________

Cell:	___________________