Page 1

REQUEST FOR BID        RETURN BY OPENING TIME TO:

Date:08/21/2013        Purchasing Division

Bid: 13-0475           RM 100 City Hall

411 West 1st Street

Duluth, MN 55802

Lund Tool House Roof Replacement

Buyer: Dennis Sears

Phone: 218-730-5003

Fax:   218-730-5922

BID OPENING, RM 100 AT 2:00 PM ON, Wednesday September 4, 2013

Note: All bids must be written, signed, and transmitted in a sealed envelope, plainly marked with the bid number, subject matter, and opening date. The City of Duluth reserves the right to split award where there is substantial savings to the city, waive informalities and to reject any and all bids. Bidder should state in proposal if bid is based on acceptance of total order. Sales tax is not to be included in the unit price. Bidder to state freight charges if, proposal is F.O.B. shipping point, freight not allowed. Low bid will not be the only consideration for award of bid. All pages must be signed or initialed by authorized bidder’s representative as indicated at the bottom of the page(s) of the request for bid.

OFFICIAL SEALED BID

Designated F.O.B. Point

Tax: Federal Excise Tax Exemption

Account No. 41-74-0056 K

----------------------------------------------------------------------

Item   Qty   U/OM          Description                 Unit     Total

No.                                                   Price    Price

----------------------------------------------------------------------

Please furnish a quote to replace the

roof on the Lund Tool House Scope and

Specification.

Vendor E-mail Address_________________________ Freight Charges    N/A

Name____________________                     Total Bid Price ________

Addr____________________               (To include any additional pages)

____________________

____________________                     Payment Terms __N/A______

By:_______________________                   F.O.B. Point  N/A.

(print title)

__________________________                   Delivery Date  N/A

(signature)    (tele#)

PROJECT SCOPE

Square foot area of roof is approximately 9836 sq. ft.

Remove and dispose of the sheeting for the entire roof area.

Remove any and all fasteners and clean purlins on any debris prior of the installation of the new roof sheeting.

Inspect the insulation and inform owner if any materials need to be replaced before installation of new roof sheeting.

Remove closures from existing perimeter metal flashing. All flashings are to be salvaged and reused with new roof system. Contractor will provide a per linear foot price for any flashing that may have to be replaced and inform the owner of any flashing that needs to be replaced.

Install new 24 gauge white prefinished exposed fasteners, corrugated roof panel with closures, gasket and sealant to create a water tight finish.

Re-install the perimeter metal flashing.

A pre-bid site visit is schedule for 9 AM, Wednesday, August 28th.

Contact person is Rob Hurd, 218-626-5130.

A bid bond is required for this project and must accompany the bid.

A current Certificate of Insurance must be on file in the purchasing office. Please see attached for insurance information.

A payment and performance bond will be required of successful bidder.