Firm Name:_________________________________     Date: September 27, 2011

Rev. Spec. #968-32C2

CITY OF DULUTH

MINIMUM SPECIFICATIONS FOR

DEMOLITION OF BUILDINGS

(Demolition Vendor Responsible for Utility Cut Offs)

Bids to meet or exceed specifications listed below.

INFORMATION

It is the intent of these specifications to describe a demolition in sufficient detail to secure bids on demolition.

Any additional, deletions or variations from the following specifications must be stated. Reason for variations and deviations must also be stated. These specifications shall be construed as minimum. Failure to complete all information on bid request form may be cause for rejection of bid.

GENERAL

The contractor shall furnish all labor, material and equipment and shall perform all services and work required to wreck and remove the listed buildings in strict accordance with the specifications and with the City of Duluth ordinances pertaining to the moving or wrecking of buildings.

All work shall be performed by mechanics skilled in demolition of all types of structures and shall be subject to approval by the Building Official or duly qualified representative.

The contractor will be required to comply with all applicable Federal, State or Local laws, regulations or ordinances and it is expressly understood and agreed that buildings indicated in this bid request may not be moved and re-erected upon some other site but are to be demolished upon and removed from the premises.

All other work under the contract shall be completed within thirty (30) working days from date of purchase order or completion of a signed contract unless otherwise authorized under terms of the bid.

The City of Duluth reserves the right to award contract in accordance with Ordinance No. 7050 which indicates the various provisions, in addition to price, which may be considered in determining the lowest responsible bidder.

Page 1 of 5

Firm Name:_________________________________  Date: September 27, 2011

Rev. Spec. #968-32C2

FILLING EXCAVATIONS

In addition to wrecking and removing the building(s), the contractor will remove all exterior and interior foundation walls, columns, piers, beams or other projections. All building and building service piping, heating plants, or other fixtures, furniture, partitions, steps, rubbish, or other debris shall be removed from the premises. All combustible debris shall be removed from the premises. Concrete stairs and walks shall be removed from the premises and those areas graded. Excavation shall be filled to a plane twelve (12) inches below the adjacent undisturbed ground surface with

common fill material such as gravel or coarse sand which may include some clay and rocks with a maximum dimension of four (4) inches. The top twelve (12) inches shall be backfilled with compact coarse granular material, with sufficient binder to permit adequate consolidation and stabilization of surface, and finished to conform to a minimum grade of 2% for drainage to adjacent undisturbed ground unless otherwise authorized by the Building Inspection Division.

UTILITY SERVICE

Any sewer, water and gas services cut off shall be the responsibility of the awarded demolition vendor/subcontractor in accordance with the regulations of the City of Duluth and under their supervision. Telephone and electric service shall be terminated under the supervision of the utility company owning the service. Adherence to the City of Duluth Specifications for cutting off and/or plugging of old unused water wells, water, gas and sewer services shall be required.

Before commencing work, contractor will be required to check and make sure that all utility services have been shut off or disconnected at the source.

Adequate notice shall be given to the Building Inspection Division prior to the start of demolition of each unit included in the bid.

DISPOSAL OF SOLID WASTE

All disposal waste materials must be disposed of at a site approved by the MPCA and WLSSD.

Page 2 of 5

Firm Name: _________________________________   Date: September 27, 2011

Rev. Spec. #968-32C2

REMOVAL AND SALVAGE OF EXISTING BUILDINGS

- At the time the Contractor moves onto the demolition site to begin demolition, he

shall have a right of salvage to all materials that exist because of the demolition of the structure under the Contract, subject to all the provisions of the contract and the following:

  - Contractor shall notify the Building Inspection Division if he finds on the site:

    - Personal property which is obviously of considerably more value than salvage value.

    - Personal property which he knows or has reason to believe belongs to a third party.

    - Motor vehicles.

  - Only such property may be salvaged by the Contractor as is owned by the

land owner and in the event of any doubt respecting the ownership of any

particular property, the Contractor shall request from the landowner a

written statement respecting its ownership.

  - Personal property of the third persons or occupants of buildings on the

site shall not become the property of the Contractor.

  - Any salvage workers authorized by the contractor to be on the property

shall be considered as subcontractors for indemnification purposes.

- Unless otherwise specified, no dwelling structure shall be removed from the

premises as a whole, or in substantially whole condition, but all such buildings

shall be demolished on the premises.

TREES, SHRUBBERY, SOD

No trees on the property shall be removed without permission. Care shall be exercised that all trees, shrubbery and sod on adjoining property will not be damaged.

SAFETY AND CLEANUP

The structure shall be demolished upon the site. All combustibles and scrap material shall be removed by the Contractor. All basement floors shall be broken up sufficiently to provide drainage. Maximum size pieces shall not exceed one square yard in area.

Under no circumstances shall dust and debris be allowed to blow or scatter from the area as a result of the demolition operation. If necessary, the contractor will be required to maintain a source of water to dampen and water down the structure as the demolition operation proceeds.

Page 3 of 5

Firm: ______________________________________   Date: September 27, 2011

Rev. Spec. #968-32C2

Damage to sidewalks, curb and gutter, street paving and utility structures shall be avoided on or adjoining the site. Any damage caused by the operations shall be required at the expense of the Contractor prior to final contract payment.

LICENSES AND PERMITS

All expense and cost of permits arising from or in conjuncture with the performance or the provision of these specifications shall be borne by the successful bidder.

The successful bidder shall possess or obtain all required permits and licenses and pay the prescribed fees. No Purchase orders or contracts will be issued until the successful contractor applies for, receives and pays for the require demolition permit.

BONDING

After award of contract and before commencing work, the contractor must submit a corporate surety performance bond (city’s form) in the amount equal to the amount of their bid, or in lieu of such corporate surety bond, a check in an amount equal to the amount of their bid, drawn in favor of the City of Duluth on a bank authorized to transact business in the State of Minnesota. The Bond must be approved by the city attorney prior to commencement of work. Bidder should be aware that permits required to wreck structures or obstruct streets in the City of Duluth require additional bonds.

INSURANCE

Contractor shall provide Public Liability and Automobile Liability Insurance with limits not less than $1,500,000 Single Limit, and twice the limit provided when a claim arises out of release or threatened release of a hazardous substance; shall be with a company approved by the City of Duluth; shall provide for the following; Liability for Premises, Operations, Completed Operations; Independent Contractors and Contractual Liability.

City of Duluth shall be named as Additional Insured under Public liability, *Excess/Umbrella Liability, and Automobile Liability, or as an alternate, Contractor may provide Owners-Contractor Protective policy, naming itself and the City of Duluth. Contractor shall also provide evidence of Statutory Minnesota Workman’s compensation Insurance. Contractor to provide Certificate of Insurance evidencing such coverage with 30-days notice of cancellation non-renewal or material change provisions included. The City of Duluth does not represent or guarantee that these types or limits or coverage are adequate to protect the Contractor’s interests and liabilities. If a Certificate of Insurance is provided, the form of the certificate shall contain an unconditional requirement that the insurer must notify the City without fail not less than 30 days prior to any cancellation,

Page 4 of 5

Firm: _______________________________________   Date: September 27, 2011

Rev. Spec. #968-32C2

non-renewal or modification of policy or coverage’s evidence by said certificate and shall further provide that failure to give such notice to the City will render any such change or changes in said policy or coverage ineffective as against the City.

The use of an “Accord” form as a certificate of insurance shall be accompanied by two forms:

1. ISO Additional Insured Endorsment (CG 2010 pre 2004)

2. Notice of Cancellation Endorsement (IL 7002) or equivalent, as approved by the City of Duluth Attorney’s Office.

(Inquire of Purchasing, 218-730-4340, for copies if needed)

*An umbrella policy with a “following form” provision is acceptable if written verification is provided that the underlying policy names the City of Duluth as an additional insured.

RIGHT OF THE CITY TO DO THE WORK

If the successful bidder should neglect to prosecute the work properly, or fail to perform any provision of the contract, the city, after three (3) days written notice to the successful bidder, may without prejudice to any other remedy the city may have, make good such deficiencies and may deduct the cost thereof from the payment then or thereafter due the successful bidder, provided, however, that the Building Official shall approve such action and the amount charged to the successful bidder.

HAZARDOUS WASTE

Abatement of asbestos and regulated waste and stabilization or lead paint is to be bid as a separate pre-demolition project. Demolition will not start until asbestos, regulated waste, and lead have been abated and stabilized as required by law.

MPCA NOTITIFICATION OF INTENT TO PERFORM DEMOLITION

Contractor must properly complete this form and any/all other documents required by City, State, and federal regulations and forward as required. Copies of all forms shall also be forwarded to the City of Duluth Building Safety Office.

GOPHER STATE ONE-CALL

Contractor MUST call 800-262-1166 and comply with all Gopher State One-Call requirements.

Page 5 of 5