Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 01 7300

EXECUTION

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes general administrative and procedural requirements governing execution of the Work including, but not limited to, the following:

          - Construction layout.

          - Field engineering and surveying.

          - Installation of the Work.

          - Cutting and patching.

          - Coordination of Owner-installed products.

          - Progress cleaning.

          - Starting and adjusting.

          - Protection of installed construction.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 01 1000 "Summary" for limits on use of Project site.

          - Section 01 3000 "Administrative Procedures" for submitting surveys.

Revise first subparagraph below to suit Project.

          - Section 01 7700 "Closeout Procedures" for submitting final property survey with Project Record Documents, recording of Owner-accepted deviations from indicated lines and levels, and final cleaning.

Retain first subparagraph below if alterations are required and Division 02 Section "Selective Structure Demolition" is used.

          - Section 02 4119 "Selective Demolition" for demolition and removal of selected portions of the building.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

        - Cutting:  Removal of in-place construction necessary to permit installation or performance of other work.

        - Patching:  Fitting and repair work required to restore construction to original conditions after installation of other work.

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.  Contractor's professional engineer may be capable of performing certain field-engineering tasks; however, for more complex tasks and for certified surveys, qualified land surveyors are more frequently required.

        - Qualification Data:  For land surveyor.

        - Certificates:  Submit certificate signed by land surveyor certifying that location and elevation of improvements comply with requirements.

Retain "Cutting and Patching Plan" Paragraph below only if required by Owner or by special Project requirements; review of a contractor's cutting and patching plan may be beyond scope of architectural services due to Contractor responsibility for means and methods of construction.  Revise to include other submittal requirements.

Retain "Landfill Receipts" Paragraph below if not retaining requirement for landfill receipts in Section 017419 "Construction Waste Management and Disposal."

        - Landfill Receipts:  Submit copy of receipts issued by a landfill facility, licensed to accept hazardous materials, for hazardous waste disposal.

Verify local practices when selecting type of entity to provide surveying services for Project.  See Evaluations.

Retain "Final Property Survey" Paragraph below if final property survey is required as part of this Contract.  Verify number of copies with Owner.

        - Final Property Survey:  Submit one electronic copy showing the Work performed and record survey data.

      - QUALITY ASSURANCE

Revise this article to comply with local requirements.  See Evaluations.

        - Land Surveyor Qualifications:  A professional land surveyor who is legally qualified to practice in jurisdiction where Project is located and who is experienced in providing land-surveying services of the kind indicated.

        - Cutting and Patching:  Comply with requirements for and limitations on cutting and patching of construction elements.

          - Structural Elements:  When cutting and patching structural elements, notify Architect of locations and details of cutting and await directions from Architect before proceeding.  Shore, brace, and support structural elements during cutting and patching.  Do not cut and patch structural elements in a manner that could change their load-carrying capacity or increase deflection

Retain first subparagraph below if providing a list of structural elements requiring review prior to cutting and patching.  Revise to suit Project.

          - Operational Elements:  Do not cut and patch operating elements and related components in a manner that results in reducing their capacity to perform as intended or that results in increased maintenance or decreased operational life or safety.  Operational elements include the following:

List below is an example only.  Revise to suit Project.  With advice of counsel, delete below if Architect's review is not required.  If list is deleted, delete option in "Operational Elements" Subparagraph above.

            - Fire-detection and -alarm systems.

            - Electrical wiring systems.

          - Other Construction Elements:  Do not cut and patch other construction elements or components in a manner that could change their load-carrying capacity, that results in reducing their capacity to perform as intended, or that results in increased maintenance or decreased operational life or safety.  Other construction elements include but are not limited to the following:

List below is an example only.  Revise to suit Project.  With advice of counsel, delete below if Architect's review is not required.  If list is deleted, delete option in "Other Construction Elements" Subparagraph above.

            - Water, moisture, or vapor barriers.

            - Membranes and flashings.

            - Equipment supports.

          - Visual Elements:  Do not cut and patch construction in a manner that results in visual evidence of cutting and patching.  Do not cut and patch exposed construction in a manner that would, in Architect's opinion, reduce the building's aesthetic qualities.  Remove and replace construction that has been cut and patched in a visually unsatisfactory manner.

Usually delete "Cutting and Patching Conference" Paragraph below.  Revise if cutting and patching are extensive or if careful coordination between several trades is necessary to avoid conflicts.

        - Cutting and Patching Conference:  Before proceeding, meet at Project site with parties involved in cutting and patching, including mechanical and electrical trades.  Review areas of potential interference and conflict.  Coordinate procedures and resolve potential conflicts before proceeding.

        - Manufacturer's Installation Instructions:  Obtain and maintain on-site manufacturer's written recommendations and instructions for installation of products and equipment.

- PRODUCTS

      - MATERIALS

        - In-Place Materials:  Use materials for patching identical to in-place materials.  For exposed surfaces, use materials that visually match in-place adjacent surfaces to the fullest extent possible.

          - If identical materials are unavailable or cannot be used, use materials that, when installed, will provide a match acceptable to Architect for the visual and functional performance of in-place materials.

- EXECUTION

      - EXAMINATION

Revise "Existing Conditions" Paragraph below to suit Project.  Verify with authorities having jurisdiction and utility companies.  Coordinate with Section 330500 "Common Work Results for Utilities."

        - Existing Conditions:  The existence and location of underground and other utilities and construction indicated as existing are not guaranteed.  Before beginning sitework, investigate and verify the existence and location of underground utilities, and other construction affecting the Work.

          - Before construction, verify the location and invert elevation at points of connection of sanitary sewer, storm sewer, and water-service piping; underground electrical services, and other utilities.

          - Furnish location data for work related to Project that must be performed by public utilities serving Project site.

        - Examination and Acceptance of Conditions:  Before proceeding with each component of the Work, examine substrates, areas, and conditions, with Installer or Applicator present where indicated, for compliance with requirements for installation tolerances and other conditions affecting performance.  Record observations.

          - Examine roughing-in for mechanical and electrical systems to verify actual locations of connections before equipment and fixture installation.

          - Examine walls, floors, and roofs for suitable conditions where products and systems are to be installed.

          - Verify compatibility with and suitability of substrates, including compatibility with existing finishes or primers.

Retain "Written Reports" Subparagraph below if required.

        - Proceed with installation only after unsatisfactory conditions have been corrected.  Proceeding with the Work indicates acceptance of surfaces and conditions.

      - PREPARATION

Revise "Existing Utility Information" Paragraph below to suit Project.

        - Existing Utility Information:  Furnish information to [local utility] [Owner] that is necessary to adjust, move, or relocate existing utility structures, utility poles, lines, services, or other utility appurtenances located in or affected by construction.  Coordinate with authorities having jurisdiction.

        - Field Measurements:  Take field measurements as required to fit the Work properly.  Recheck measurements before installing each product.  Where portions of the Work are indicated to fit to other construction, verify dimensions of other construction by field measurements before fabrication.  Coordinate fabrication schedule with construction progress to avoid delaying the Work.

        - Space Requirements:  Verify space requirements and dimensions of items shown diagrammatically on Drawings.

Revise "Review of Contract Documents and Field Conditions" Paragraph below to suit Project and Owner's special requirements.

        - Review of Contract Documents and Field Conditions:  Immediately on discovery of the need for clarification of the Contract Documents caused by differing field conditions outside the control of Contractor, submit a request for information to Architect according to requirements in Section 01 3000 "Administrative Procedures."

      - CONSTRUCTION LAYOUT

        - Verification:  Before proceeding to lay out the Work, verify layout information shown on Drawings, in relation to the property survey and existing benchmarks.  If discrepancies are discovered, notify Architect promptly.

        - General:  Engage a land surveyor to lay out the Work using accepted surveying practices.

          - Establish benchmarks and control points to set lines and levels at each story of construction and elsewhere as needed to locate each element of Project.

          - Establish limits on use of Project site.

          - Establish dimensions within tolerances indicated.  Do not scale Drawings to obtain required dimensions.

          - Inform installers of lines and levels to which they must comply.

          - Check the location, level and plumb, of every major element as the Work progresses.

          - Notify Architect when deviations from required lines and levels exceed allowable tolerances.

          - Close site surveys with an error of closure equal to or less than the standard established by authorities having jurisdiction.

        - Site Improvements:  Locate and lay out site improvements, including pavements, grading, fill and topsoil placement, utility slopes, and rim and invert elevations.

        - Building Lines and Levels:  Locate and lay out control lines and levels for structures, building foundations, column grids, and floor levels, including those required for mechanical and electrical work.  Transfer survey markings and elevations for use with control lines and levels.  Level foundations and piers from two or more locations.

        - Record Log:  Maintain a log of layout control work.  Record deviations from required lines and levels.  Include beginning and ending dates and times of surveys, weather conditions, name and duty of each survey party member, and types of instruments and tapes used.  Make the log available for reference by Architect.

Insert special requirements for laying out the Work to suit Project.

      - FIELD ENGINEERING

Revise "Identification" Paragraph below to suit Project or delete.  Requirement exceeds that required in AIA Document A201, which only requires Owner to furnish a survey.

        - Identification:  Owner will identify existing benchmarks, control points, and property corners.

        - Reference Points:  Locate existing permanent benchmarks, control points, and similar reference points before beginning the Work.  Preserve and protect permanent benchmarks and control points during construction operations.

          - Do not change or relocate existing benchmarks or control points without prior written approval of Architect.  Report lost or destroyed permanent benchmarks or control points promptly.  Report the need to relocate permanent benchmarks or control points to Architect before proceeding.

          - Replace lost or destroyed permanent benchmarks and control points promptly.  Base replacements on the original survey control points.

Insert additional requirements for preserving permanent benchmarks and control points.

        - Benchmarks:  Establish and maintain a minimum of two permanent benchmarks on Project site, referenced to data established by survey control points.  Comply with authorities having jurisdiction for type and size of benchmark.

          - Record benchmark locations, with horizontal and vertical data, on Project Record Documents.

          - Where the actual location or elevation of layout points cannot be marked, provide temporary reference points sufficient to locate the Work.

          - Remove temporary reference points when no longer needed.  Restore marked construction to its original condition.

Retain "Certified Survey" Paragraph below if required by Owner or authorities having jurisdiction.

        - Certified Survey:  On completion of foundation walls, major site improvements, and other work requiring field-engineering services, prepare a certified survey showing dimensions, locations, angles, and elevations of construction and sitework.

Retain "Final Property Survey" Paragraph below if Contractor is required to provide final property survey.

        - Final Property Survey:  Engage a land surveyor to prepare a final property survey showing significant features (real property) for Project.  Include on the survey a certification, signed by land surveyor, that principal metes, bounds, lines, and levels of Project are accurately positioned as shown on the survey.

          - Show boundary lines, monuments, streets, site improvements and utilities, existing improvements and significant vegetation, adjoining properties, acreage, grade contours, and the distance and bearing from a site corner to a legal point.

Delete "Recording" Subparagraph below if Owner will record survey.

          - Recording:  At Substantial Completion, have the final property survey recorded by or with authorities having jurisdiction as the official "property survey."

Insert other special field-engineering requirements, such as damage surveys, settlement surveys and reports, environmental impact surveys, and similar requirements, as needed.

      - INSTALLATION

        - General:  Locate the Work and components of the Work accurately, in correct alignment and elevation, as indicated.

          - Make vertical work plumb and make horizontal work level.

          - Where space is limited, install components to maximize space available for maintenance and ease of removal for replacement.

          - Conceal pipes, ducts, and wiring in finished areas unless otherwise indicated.

          - Maintain minimum headroom clearance of 96 inches (2440 mm) in occupied spaces.

        - Comply with manufacturer's written instructions and recommendations for installing products in applications indicated.

        - Install products at the time and under conditions that will ensure the best possible results.  Maintain conditions required for product performance until Substantial Completion.

        - Conduct construction operations so no part of the Work is subjected to damaging operations or loading in excess of that expected during normal conditions of occupancy.

        - Sequence the Work and allow adequate clearances to accommodate movement of construction items on site and placement in permanent locations.

        - Tools and Equipment:  Do not use tools or equipment that produce harmful noise levels.

        - Templates:  Obtain and distribute to the parties involved templates for work specified to be factory prepared and field installed.  Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing products to comply with indicated requirements.

        - Attachment:  Provide blocking and attachment plates and anchors and fasteners of adequate size and number to securely anchor each component in place, accurately located and aligned with other portions of the Work.  Where size and type of attachments are not indicated, verify size and type required for load conditions.

          - Mounting Heights:  Where mounting heights are not indicated, mount components at heights directed by Architect.

          - Allow for building movement, including thermal expansion and contraction.

          - Coordinate installation of anchorages.  Furnish setting drawings, templates, and directions for installing anchorages, including sleeves, concrete inserts, anchor bolts, and items with integral anchors, that are to be embedded in concrete or masonry.  Deliver such items to Project site in time for installation.

        - Joints:  Make joints of uniform width.  Where joint locations in exposed work are not indicated, arrange joints for the best visual effect.  Fit exposed connections together to form hairline joints.

        - Hazardous Materials:  Use products, cleaners, and installation materials that are not considered hazardous.

Insert additional installation requirements to suit Project.  See Evaluations.

      - CUTTING AND PATCHING

        - Cutting and Patching, General:  Employ skilled workers to perform cutting and patching.  Proceed with cutting and patching at the earliest feasible time, and complete without delay.

          - Cut in-place construction to provide for installation of other components or performance of other construction, and subsequently patch as required to restore surfaces to their original condition.

        - Existing Warranties:  Remove, replace, patch, and repair materials and surfaces cut or damaged during installation or cutting and patching operations, by methods and with materials so as not to void existing warranties.

        - Temporary Support:  Provide temporary support of work to be cut.

        - Protection:  Protect in-place construction during cutting and patching to prevent damage.  Provide protection from adverse weather conditions for portions of Project that might be exposed during cutting and patching operations.

Retain "Adjacent Occupied Areas" and "Existing Utility Services and Mechanical/Electrical Systems" paragraphs below where Owner continues to occupy other portions of an existing facility.

        - Adjacent Occupied Areas:  Where interference with use of adjoining areas or interruption of free passage to adjoining areas is unavoidable, coordinate cutting and patching according to requirements in Section 011000 "Summary."

        - Existing Utility Services and Mechanical/Electrical Systems:  Where existing services/systems are required to be removed, relocated, or abandoned, bypass such services/systems before cutting to minimize interruption to occupied areas.

        - Cutting:  Cut in-place construction by sawing, drilling, breaking, chipping, grinding, and similar operations, including excavation, using methods least likely to damage elements retained or adjoining construction.  If possible, review proposed procedures with original Installer; comply with original Installer's written recommendations.

          - In general, use hand or small power tools designed for sawing and grinding, not hammering and chopping.  Cut holes and slots neatly to minimum size required, and with minimum disturbance of adjacent surfaces.  Temporarily cover openings when not in use.

          - Finished Surfaces:  Cut or drill from the exposed or finished side into concealed surfaces.

          - Concrete and Masonry:  Cut using a cutting machine, such as an abrasive saw or a diamond-core drill.

          - Excavating and Backfilling:  Comply with requirements in applicable Sections where required by cutting and patching operations.

Revise "Mechanical and Electrical Services" Subparagraph below to suit Project.

          - Mechanical and Electrical Services:  Cut off pipe or conduit in walls or partitions to be removed.  Cap, valve, or plug and seal remaining portion of pipe or conduit to prevent entrance of moisture or other foreign matter after cutting.

Retain subparagraph below if required to prevent multiple cutting and patching in the same area.  Add specific requirements for multiple contracts and special conditions requiring coordination.

          - Proceed with patching after construction operations requiring cutting are complete.

        - Patching:  Patch construction by filling, repairing, refinishing, closing up, and similar operations following performance of other work.  Patch with durable seams that are as invisible as practicable.  Provide materials and comply with installation requirements specified in other Sections, where applicable.

          - Inspection:  Where feasible, test and inspect patched areas after completion to demonstrate physical integrity of installation.

          - Exposed Finishes:  Restore exposed finishes of patched areas and extend finish restoration into retained adjoining construction in a manner that will minimize evidence of patching and refinishing.

            - Clean piping, conduit, and similar features before applying paint or other finishing materials.

            - Restore damaged pipe covering to its original condition.

Insert specific refinishing requirements for floors, walls, and ceilings.  Revise "Floors and Walls" Subparagraph below to suit Project.

          - Floors and Walls:  Where walls or partitions that are removed extend one finished area into another, patch and repair floor and wall surfaces in the new space.  Provide an even surface of uniform finish, color, texture, and appearance.  Remove in-place floor and wall coverings and replace with new materials, if necessary, to achieve uniform color and appearance.

            - Where patching occurs in a painted surface, prepare substrate and apply primer and intermediate paint coats appropriate for substrate over the patch, and apply final paint coat over entire unbroken surface containing the patch.  Provide additional coats until patch blends with adjacent surfaces.

Revise "Ceilings" Subparagraph below to suit Project or delete.

          - Ceilings:  Patch, repair, or rehang in-place ceilings as necessary to provide an even-plane surface of uniform appearance.

          - Exterior Building Enclosure:  Patch components in a manner that restores enclosure to a weathertight condition and ensures thermal and moisture integrity of building enclosure.

        - Cleaning:  Clean areas and spaces where cutting and patching are performed.  Remove paint, mortar, oils, putty, and similar materials from adjacent finished surfaces.

Insert specific installation requirements if not specified elsewhere.  Specific installation requirements are better specified in individual Sections.

      - PROGRESS CLEANING

This article refers to regular cleaning operations conducted while construction is in progress.  Requirements for final cleaning before Substantial Completion are included in Section 017700 "Closeout Procedures."

        - General:  Clean Project site and work areas daily, including common areas.  Enforce requirements strictly.  Dispose of materials lawfully.

          - Comply with requirements in NFPA 241 for removal of combustible waste materials and debris.

          - Do not hold waste materials more than seven days during normal weather or three days if the temperature is expected to rise above 80 deg F (27 deg C).

          - Containerize hazardous and unsanitary waste materials separately from other waste.  Mark containers appropriately and dispose of legally, according to regulations.

            - Use containers intended for holding waste materials of type to be stored.

Retain subparagraph below for projects involving multiple contracts.

          - Coordinate progress cleaning for joint-use areas where Contractor and other contractors are working concurrently.

        - Site:  Maintain Project site free of waste materials and debris.

        - Work Areas:  Clean areas where work is in progress to the level of cleanliness necessary for proper execution of the Work.

          - Remove liquid spills promptly.

          - Where dust would impair proper execution of the Work, broom-clean or vacuum the entire work area, as appropriate.

        - Installed Work:  Keep installed work clean.  Clean installed surfaces according to written instructions of manufacturer or fabricator of product installed, using only cleaning materials specifically recommended.  If specific cleaning materials are not recommended, use cleaning materials that are not hazardous to health or property and that will not damage exposed surfaces.

        - Concealed Spaces:  Remove debris from concealed spaces before enclosing the space.

        - Exposed Surfaces in Finished Areas:  Clean exposed surfaces and protect as necessary to ensure freedom from damage and deterioration at time of Substantial Completion.

        - Waste Disposal:  Do not bury or burn waste materials on-site.  Do not wash waste materials down sewers or into waterways.  Comply with waste disposal requirements in Section 017419 "Construction Waste Management and Disposal."

Three paragraphs below reduce or eliminate the need for similar provisions in other Sections.  Insert other provisions needed because of unusual Project conditions.  Specify unusual provisions for specific work in the individual Section.

        - During handling and installation, clean and protect construction in progress and adjoining materials already in place.  Apply protective covering where required to ensure protection from damage or deterioration at Substantial Completion.

        - Clean and provide maintenance on completed construction as frequently as necessary through the remainder of the construction period.  Adjust and lubricate operable components to ensure operability without damaging effects.

If necessary, revise "Limiting Exposures" Paragraph below by inserting a list of exposures.  See Evaluations.

        - Limiting Exposures:  Supervise construction operations to assure that no part of the construction, completed or in progress, is subject to harmful, dangerous, damaging, or otherwise deleterious exposure during the construction period.

Special cleaning provisions are often required on projects that include alterations to existing buildings.  See Evaluations.

      - STARTING AND ADJUSTING

        - Start equipment and operating components to confirm proper operation.  Remove malfunctioning units, replace with new units, and retest.

        - Adjust equipment for proper operation.  Adjust operating components for proper operation without binding.

        - Test each piece of equipment to verify proper operation.  Test and adjust controls and safeties.  Replace damaged and malfunctioning controls and equipment.

        - Manufacturer's Field Service:  Comply with qualification requirements in Section 01 4000 "Quality Requirements."

      - PROTECTION OF INSTALLED CONSTRUCTION

Revise first paragraph below to suit Project conditions and special Owner requirements.

        - Provide final protection and maintain conditions that ensure installed Work is without damage or deterioration at time of Substantial Completion.

        - Comply with manufacturer's written instructions for temperature and relative humidity.

END OF SECTION 01 7300