Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 07 2500

WEATHER BARRIERS

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Building wrap.

          - Flexible flashing.

      - ACTION SUBMITTALS

        - Product Data:  For each type of product.

          - For building wrap, include data on air and water-vapor permeance based on testing according to referenced standards.

- PRODUCTS

      - WATER-RESISTIVE BARRIER

Option in "Building Paper" Paragraph below is based on "Super Jumbo Tex 60 Minute" by Fortifiber.

Note that ASTM standard referenced in "Building Wrap" Paragraph below is for air barriers for low-rise buildings.  This standard allows 15 times as much air infiltration as the limit for air barriers in the Massachusetts State Building Code.

        - Building Wrap:  ASTM E 1677, Type I air barrier; with flame-spread and smoke-developed indexes of less than 25 and 450, respectively, when tested according to ASTM E 84; UV stabilized; and acceptable to authorities having jurisdiction.

Retain "Products" Subparagraph and list of manufacturers and products below to require specific products or a comparable product from other manufacturers.

          - Products:  Subject to compliance with requirements, available products that may be incorporated into the Work include, but are not limited to, the following:

            - DuPont (E. I. du Pont de Nemours and Company); Tyvek CommercialWrap.

Coordinate permeance value retained or inserted in "Water-Vapor Permeance" Subparagraph below with products retained.  Consult manufacturers' literature.

          - Water-Vapor Permeance:  As required by code or as recommended by weather barrier manufacturer for project location.

          - Allowable UV Exposure Time:  Not less than three months.

        - Building-Wrap Tape:  Pressure-sensitive plastic tape recommended by building-wrap manufacturer for sealing joints and penetrations in building wrap.

      - MISCELLANEOUS MATERIALS

        - Flexible Flashing: Composite, self-adhesive, flashing product consisting of a pliable, compound, bonded to a high-density polyethylene film, aluminum foil, or spunbonded polyolefin to produce an overall thickness of not less than 0.030 inch (0.8 mm).

Retain "Products" Subparagraph and list of manufacturers and products below to require specific products or a comparable product from other manufacturers.

          - Products: Subject to compliance with requirements, available products that may be incorporated into the Work include, but are not limited to, the following:

Products in first four subparagraphs below are butyl rubber products.

            - DuPont (E. I. du Pont de Nemours and Company); DuPont Flashing Tape.

            - Grace Construction Products, a unit of W. R. Grace & Co. - Conn.; Vycor Butyl Self Adhered Flashing.

        - Primer for Flexible Flashing:  Product recommended by manufacturer of flexible flashing for substrate.

        - Nails and Staples:  ASTM F 1667.

- EXECUTION

      - WATER-RESISTIVE BARRIER INSTALLATION

        - Cover exposed exterior surface of sheathing with water-resistive barrier securely fastened to framing immediately after sheathing is installed.

        - Cover sheathing with water-resistive barrier as follows:

          - Cut back barrier 1/2 inch (13 mm) on each side of the break in supporting members at expansion- or control-joint locations.

          - Apply barrier to cover vertical flashing with a minimum 4-inch (100-mm) overlap unless otherwise indicated.

        - Building Wrap:  Comply with manufacturer's written instructions.

          - Seal seams, edges, fasteners, and penetrations with tape.

          - Extend into jambs of openings and seal corners with tape.

      - FLEXIBLE FLASHING INSTALLATION

        - Apply flexible flashing where indicated to comply with manufacturer's written instructions.

          - Prime substrates as recommended by flashing manufacturer.

          - Lap seams and junctures with other materials at least 4 inches (100 mm) except that at flashing flanges of other construction, laps need not exceed flange width.

          - Lap flashing over water-resistive barrier at bottom and sides of openings.

Retain first subparagraph below if head flashing is used; see Evaluations.

          - Lap water-resistive barrier over flashing at heads of openings.

          - After flashing has been applied, roll surfaces with a hard rubber or metal roller to ensure that flashing is completely adhered to substrates.

END OF SECTION 07 2500