- SECTION 08 7100

- DOOR HARDWARE

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes commercial door hardware for the following:

          - Swinging doors.

        - Related Sections:

          - Section 08 1100 – “Steel Doors and Frames”.

      - SUBMITTALS

        - Product Data: Manufacturer's product data sheets including installation details, material descriptions, dimensions of individual components and profiles, operational descriptions and finishes.

        - Door Hardware Schedule: Prepared by or under the supervision of supplier, detailing fabrication and assembly of door hardware, as well as procedures and diagrams. Coordinate the final Door Hardware Schedule with doors, frames, and related work to ensure proper size, thickness, hand, function, and finish of door hardware.

          - Format: Comply with scheduling sequence and vertical format in DHI's "Sequence and Format for the Hardware Schedule."

          - Organization: Organize the Door Hardware Schedule into door hardware sets indicating complete designations of every item required for each door or opening. Organize door hardware sets in same order as in the Door Hardware Sets at the end of Part 3. Submittals that do not follow the same format and order as the Door Hardware Sets will be rejected and subject to resubmission.

          - Content: Include the following information:

            - Type, style, function, size, label, hand, and finish of each door hardware item.

            - Manufacturer of each item.

            - Fastenings and other pertinent information.

            - Location of door hardware set, cross-referenced to Drawings, both on floor plans and in door and frame schedule.

            - Explanation of abbreviations, symbols, and codes contained in schedule.

            - Mounting locations for door hardware.

            - Door and frame sizes and materials.

          - Submittal Sequence: Submit the final Door Hardware Schedule at earliest possible date, particularly where approval of the Door Hardware Schedule must precede fabrication of other work that is critical in the Project construction schedule. Include Product Data, Samples, Shop Drawings of other work affected by door hardware, and other information essential to the coordinated review of the Door Hardware Schedule.

        - Keying Schedule: Prepared under the supervision of the Owner, separate schedule detailing final keying instructions for locksets and cylinders in writing. Include keying system explanation, door numbers, key set symbols, hardware set numbers and special instructions. Owner to approve submitted keying schedule prior to the ordering of permanent cylinders.

      - Informational Submittals

        - Qualification Data: For Installer.

        - Product Test Reports:  For compliance with accessibility requirements, based on evaluation of comprehensive tests performed by manufacturer and witnessed by a qualified testing agency, for door hardware on doors located in accessible routes.

        - Operating and Maintenance Manuals: Provide manufacturers operating and maintenance manuals for each item comprising the complete door hardware installation in quantity as required in Division 01, Closeout Submittals. The manual to include the name, address, and contact information of the manufacturers providing the hardware and their nearest service representatives. The final copies delivered after completion of the installation test to include "as built" modifications made during installation, checkout, and acceptance.

        - Warranties and Maintenance: Special warranties and maintenance agreements specified in this Section.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For each type of door hardware to include in maintenance manuals. Include final hardware and keying schedule.

      - QUALITY ASSURANCE

        - Installer Qualifications: Installers, trained by the primary product manufacturers, with a minimum 3 years documented experience installing both standard and electrified builders hardware similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

        - Door Hardware Supplier Qualifications: Experienced commercial door hardware distributors with a minimum 5 years documented experience supplying both mechanical and electromechanical hardware installations comparable in material, design, and extent to that indicated for this Project. Supplier recognized as a factory direct distributor in good standing by the manufacturers of the primary materials with a warehousing facility in Project's vicinity. Supplier to have on staff a certified Architectural Hardware Consultant (AHC) available during the course of the Work to consult with Contractor, Architect, and Owner concerning both standard and electromechanical door hardware and keying.

          - Scheduling Responsibility: Preparation of door hardware and keying schedules.

        - Source Limitations: Obtain each type and variety of Door Hardware specified in this Section from a single source, qualified supplier unless otherwise indicated.

        - Regulatory Requirements: Comply with NFPA 80, NFPA 101 and ANSI A117.1 requirements and guidelines as directed in the model building code including, but not limited to, the following:

          - Where indicated to comply with accessibility requirements, comply with Americans with Disabilities Act (ADA), "Accessibility Guidelines for Buildings and Facilities (ADAAG)," ANSI A117.1 as follows:

            - Handles, Pulls, Latches, Locks, and other Operating Devices: Shape that is easy to grasp with one hand and does not require tight grasping, tight pinching, or twisting of the wrist.

            - Door Closers: Comply with the following maximum opening-force requirements indicated:

            - Thresholds: Not more than 1/2 inch high. Bevel raised thresholds with a slope of not more than 1:2.

        - Each unit to bear third party permanent label demonstrating compliance with the referenced standards.

        - At completion of installation, provide written documentation that components were applied to manufacturer's instructions and recommendations and according to approved schedule.

      - DELIVERY, STORAGE, AND HANDLING

        - Inventory door hardware on receipt and provide secure lock-up and shelving for door hardware delivered to Project site. Do not store electronic access control hardware, software or accessories at Project site without prior authorization.

        - Tag each item or package separately with identification related to the final Door Hardware Schedule, and include basic installation instructions with each item or package.

        - Deliver, as applicable, permanent keys, cylinders, cores, access control credentials, software and related accessories directly to Owner via registered mail or overnight package service. Instructions for delivery to the Owner shall be established at the "Keying Conference".

      - COORDINATION

        - Templates: Obtain and distribute to the parties involved templates for doors, frames, and other work specified to be factory prepared for installing standard and electrified hardware. Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing hardware to comply with indicated requirements.

        - Door and Frame Preparation: Coordinate preparation of steel doors and frames indicated in Related Division 08 Sections as required for the installation of the specified hardware.

      - WARRANTY

        - Warranty Period: Written warranty, executed by manufacturer(s), agreeing to repair or replace components of standard and electrified door hardware that fails in materials or workmanship within specified warranty period after final acceptance by the Owner. Failures include, but are not limited to, the following:

          - Structural failures including excessive deflection, cracking, or breakage.

          - Faulty operation of the hardware.

          - Deterioration of metals, metal finishes, and other materials beyond normal weathering.

          - Electrical component defects and failures within the systems operation.

      - MAINTENANCE SERVICE

        - Maintenance Tools and Instructions: Furnish a complete set of specialized tools and maintenance instructions as needed for Owner's continued adjustment, maintenance, and removal and replacement of door hardware.

- PRODUCTS

      - SCHEDULED DOOR HARDWARE

        - General: Provide door hardware for each door to comply with requirements in Door Hardware Sets and each referenced section that products are to be supplied under.

          - Designations: Requirements for quantity, item, size, finish or color, grade, function, and other distinctive qualities of each type of door hardware are indicated in the Door Hardware Sets at the end of Part 3. Products are identified by using door hardware designations, as follows:

      - HANGING DEVICES

        - Hinges: ANSI/BHMA A156.1 certified butt hinges with number of hinge knuckles as specified in the Door Hardware Sets.

          - Quantity: Provide the following hinge quantity, unless otherwise indicated:

            - Three Hinges: For doors with heights 61 to 90 inches.

            - Four Hinges: For doors with heights 91 to 120 inches.

          - Hinge Size: Provide the following, unless otherwise indicated, with hinge widths sized for door thickness and clearances required:

            - Widths up to 3’0”: 4-1/2” standard or heavy weight as specified.

          - Hinge Weight and Base Material: Unless otherwise indicated, provide the following:

            - Exterior Doors: Heavy weight, non-ferrous, ball bearing or oil impregnated bearing hinges unless Hardware Sets indicate standard weight.

          - Hinge Options: Comply with the following where indicated in the Hardware Sets or on Drawings:

            - Non-removable Pins: Provide set screw in hinge barrel that, when tightened into a groove in hinge pin, prevents removal of pin while door is closed.

          - Acceptable Manufacturers:

            - Bommer Industries (BO).

            - Hager Companies (HA).

            - McKinney Products (MK).

            - Stanley Hardware (ST).

      - CYLINDERS AND KEYING

        - Source Limitations: Obtain each type of keyed cylinder and keys from the same source manufacturer as locksets and exit devices, unless otherwise indicated.

          - Acceptable Manufacturers:

            - Corbin Russwin Hardware (RU).

            - Sargent Manufacturing (SA).

            - Schlage (SC).

            - Stanley Best (BE).

        - Permanent Cores: Manufacturer's standard; finish face to match lockset; complying with the following:

          - Removable Cores: Core insert, removable by use of a special key, and for use with only the core manufacturer's cylinder and door hardware. Provide removable core (small or large format) as specified in Hardware Sets.

          - Keying System: Each type of lock and cylinders to be factory keyed. Conduct specified "Keying Conference" to define and document keying system instructions and requirements. Furnish factory cut, nickel-silver large bow permanently inscribed with a visual key control number as directed by Owner. Incorporate decisions made in keying conference.

        - Key Quantity: Provide the following minimum number of keys:

          - Top Master Key: One (1)

          - Change Keys per Cylinder: Two (2)

          - Construction Control Keys (where required): Two (2)

          - Permanent Control Keys (where required): Two (2)

        - Construction Keying: Provide construction master keyed cylinders or temporary keyed construction cores where specified. Provide construction master keys in quantity as required by project Contractor. Replace construction cores with permanent cores. Furnish permanent cores for installation as directed under specified "Keying Conference".

      - MECHANICAL LOCKS AND LATCHING DEVICES

        - Mortise Locksets, Grade 1 (Heavy Duty): ANSI/BHMA A156.13, Series 1000, Operational Grade 1 certified mortise locksets furnished in the functions as specified in the Hardware Sets. Locksets to be manufactured with a corrosion resistant, stamped 12 gauge minimum formed steel case and be field-reversible for handing without disassembly of the lock body. Lockset trim including levers and roses to be the product of a single manufacturer. Furnish with standard 2 3/4" backset, 3/4" throw anti-friction stainless steel latchbolt, and a full 1" throw stainless steel bolt for deadbolt functions.

          - Mortise locks to be certified Security Grade 1 and include vandal resistant heavy gauge escutcheon or sectional type trim.

          - Provide mortise lock bodies functionally compatible with a rose-less lever trim option.

          - Acceptable Manufacturers:

            - Corbin Russwin Hardware (RU) – ML2000 Series.

            - Sargent Manufacturing (SA) – 8200 Series.

            - Schlage (SC) – L9000 Series.

      - LOCK AND LATCH STRIKES

        - Strikes: Provide manufacturer's standard strike with strike box for each latch or lock bolt, with curved lip extended to protect frame, finished to match door hardware set, unless otherwise indicated, and as follows:

          - Flat-Lip Strikes: For locks with three-piece antifriction latchbolts, as recommended by manufacturer.

          - Extra-Long-Lip Strikes: For locks used on frames with applied wood casing trim.

        - Standards: Comply with the following:

          - Strikes for Mortise Locks and Latches: BHMA A156.13.

          - Strikes for Auxiliary Deadlocks: BHMA A156.5.

          - Dustproof Strikes: BHMA A156.16.

      - DOOR CLOSERS

        - All door closers specified herein shall meet or exceed the following criteria:

          - General: Door closers to be from one manufacturer, matching in design and style, with the same type door preparations and templates regardless of application or spring size. Closers to be non-handed with full sized covers including installation and adjusting information on inside of cover.

          - Cycle Testing:  Provide closers which have surpassed 10 million cycles in a test witnessed and verified by UL.

          - Size of Units: Comply with manufacturer's written recommendations for sizing of door closers depending on size of door, exposure to weather, and anticipated frequency of use. Where closers are indicated for doors required to be accessible to the physically handicapped, provide units complying with ANSI ICC/A117.1.

          - Closer Arms: Provide heavy duty, forged steel closer arms unless otherwise indicated in Hardware Sets.

            - Where closers are indicated to have mechanical dead-stop, provide heavy duty arms and brackets with an integral positive stop.

            - Where closers are indicated to have mechanical hold open, provide heavy duty units with an additional built-in mechanical holder assembly designed to hold open against normal wind and traffic conditions. Holder to be manually selectable to on-off position.

            - Where closers are indicated to have a cushion-type stop, provide heavy duty arms and brackets with spring stop mechanism to cushion door when opened to maximum degree.

            - Closers shall not be installed on exterior or corridor side of doors; where possible install closers on door for optimum aesthetics. Provide drop plates or other accessories as required for proper mounting.

          - Closer Covers:  Provide PVC free closer covers with a painted finish to match other hardware on the project.

          - Closer Accessories: Provide door closer accessories including custom templates, special mounting brackets, spacers and drop plates, and through-bolt or security type fasteners as specified in the door Hardware Sets.

        - Exterior Door Closers, Surface Mounted (Cam Action): ANSI/BHMA 156.4, Grade 1 certified surface mounted, high efficiency door closers with complete spring power adjustment, sizes 1 thru 6; and fully operational adjustable according to door size, frequency of use, and opening force. Closers to be of the cam and roller design, one piece cast aluminum silicon alloy body with adjustable backcheck and independently controlled valves for closing sweep and latch speed.

          - Acceptable Manufacturers:

            - Corbin Russwin (RU) - DC5000 Series.

            - Dorma Products (DO) - TS93 Series.

            - Norton Door Controls (NO) - 2800ST Series.

            - Sargent Manufacturing (SA) - 421 Series.

      - ARCHITECTURAL TRIM

        - Door Protective Trim

          - General: Door protective trim units to be of type and design as specified below or in the Hardware Sets.

          - Size: Fabricate protection plates not more than 2" less than door width (LDW). Coordinate and provide proper width and height as required where conflicting hardware dictates. Height to be as specified in the Hardware Sets.

          - Metal Protection Plates: ANSI/BHMA A156.6 certified metal protection plates (kick, armor, or mop), beveled on four edges (B4E), fabricated from the following.

            - Stainless Steel: 050-inch thick, with countersunk screw holes (CSK).

          - Fasteners: Provide manufacturer's designated fastener type as specified in the Hardware Sets.

          - Acceptable Manufacturers:

            - Burns Manufacturing (BU).

            - Hiawatha, Inc. (HI).

            - Ives (IV).

            - Rockwood Manufacturing (RO).

            - Trimco (TC).

      - ARCHITECTURAL SEALS

        - General: Thresholds, weatherstripping, and gasket seals to be of type and design as specified below or in the Hardware Sets. Provide continuous silicone or preen type weatherstrip gasketing on exterior doors. At exterior applications provide non-corrosive fasteners and elsewhere where indicated.

        - Acceptable Manufacturers:

          - National Guard (NG).

          - Pemko Manufacturing (PE).

          - Reese Enterprises, Inc. (RS).

          - Zero International (ZE).

      - FABRICATION

        - Fasteners: Provide door hardware manufactured to comply with published templates generally prepared for machine, wood, and sheet metal screws. Provide screws according to manufacturers recognized installation standards for application intended.

      - FINISHES

        - Standard: Designations used in the Hardware Sets and elsewhere indicate hardware finishes complying with ANSI/BHMA A156.18, including coordination with traditional U.S. finishes indicated by certain manufacturers for their products.

        - Provide quality of finish, including thickness of plating or coating (if any), composition, hardness, and other qualities complying with manufacturer's standards, but in no case less than specified by referenced standards for the applicable units of hardware.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Exposed Metal Finishes:		BHMA 626 / US26D = Satin Chromium plated over 						nickel, over brass or bronze base metal.

BHMA 652 / US26D = Satin Chromium plated over nickel, over Steel base metal.

BHMA 630 / US32D = Satin Stainless Steel, over stainless-steel base material.

Hinges-Exterior: 	630 / US32D

Locksets: 	626 / US26D

Closers: 	689 / AL Aluminum painted over any base material.

Pulls, Kick Plates: 	630 / US32D

Door Stops: 	630 / US32D

Thresholds:	AL – Mill finish aluminum

Weatherstrip:	AL – Mill finish aluminum

Sweep Strips: 	AL – Mill finish aluminum

Rain Drips:	Match finish of door frame.

Miscellaneous Items: 	US26D

- EXECUTION

      - EXAMINATION

        - Examine scheduled openings, with Installer present, for compliance with requirements for installation tolerances, wall and floor construction, and other conditions affecting performance.

        - Notify architect of any discrepancies or conflicts between the door schedule, door types, drawings and scheduled hardware. Proceed only after such discrepancies or conflicts have been resolved in writing.

      - PREPARATION

        - Hollow Metal Doors and Frames: Comply with ANSI/DHI A115 series.

      - INSTALLATION

        - Install each item of mechanical and electromechanical hardware and access control equipment to comply with manufacturer's written instructions and according to specifications.

          - Installers are to be trained and certified by the manufacturer on the proper installation and adjustment of life safety and security products including: hanging devices; locking devices; closing devices; and seals.

        - Mounting Heights: Mount door hardware units at heights indicated in following applicable publications, unless specifically indicated or required to comply with governing regulations:

          - Standard Steel Doors and Frames: DHI's "Recommended Locations for Architectural Hardware for Standard Steel Doors and Frames."

          - Comply with accessibility requirements, comply with ANSI A117.1 "Accessibility Guidelines for Buildings and Facilities."

          - Provide blocking in drywall partitions where wall stops or other wall mounted hardware is located.

        - Thresholds: Set thresholds for exterior and acoustical doors in full bed of sealant complying with requirements specified in Division 7 Section "Joint Sealants."

        - Storage: Provide a secure lock up for hardware delivered to the project but not yet installed. Control the handling and installation of hardware items so that the completion of the work will not be delayed by hardware losses before and after installation.

      - FIELD QUALITY CONTROL

        - Field Inspection: Supplier will perform a final inspection of installed door hardware and state in report whether work complies with or deviates from requirements, including whether door hardware is properly installed, operating and adjusted.

      - ADJUSTING

        - Initial Adjustment: Adjust and check each operating item of door hardware and each door to ensure proper operation or function of every unit. Replace units that cannot be adjusted to operate as intended. Adjust door control devices to compensate for final operation of heating and ventilating equipment and to comply with referenced accessibility requirements.

      - CLEANING AND PROTECTION

        - Protect all hardware stored on construction site in a covered and dry place. Protect exposed hardware installed on doors during the construction phase. Install any and all hardware at the latest possible time frame.

        - Clean adjacent surfaces soiled by door hardware installation.

        - Clean operating items as necessary to restore proper finish. and provide final protection and maintain conditions that ensure door hardware is without damage or deterioration at time of owner occupancy.

      - DEMONSTRATION

        - Instruct Owner's maintenance personnel to adjust, operate, and maintain mechanical and electromechanical door hardware.

      - DOOR HARDWARE SCHEDULE

        - The hardware sets represent the design intent and direction of the owner and architect. They are a guideline only and should not be considered a detailed hardware schedule. Discrepancies, conflicting hardware and missing items should be brought to the attention of the architect with corrections made prior to the bidding process. Omitted items not included in a hardware set should be scheduled with the appropriate additional hardware required for proper application and functionality.

        - Group 1 (Steel Doors and Frames 103 and 104): Each door shall have:

- 1. 	Heavy Duty Hinges 			McKinney

- 2. 	Lever Lock Set with Office Function

- 3. 	Surface Mounted Closer

- 4. 	Wall Mounted Door Stop

- 6. 	Threshold

- 7. 	Weatherstripping

- 9.	Sweep Strip Brush Type with drip 	Pemko 3452-NB

- 10.	Rain Drip (Finish to match door frame)

END OF SECTION 08 71 00