SECTION 10 1400

SIGNAGE

PART 1  GENERAL

1.01	SECTION INCLUDES

A.	Room and door signs.

1.02	RELATED REQUIREMENTS

A.	Section 26 0553 - Identification for Electrical Systems.

1.03	REFERENCE STANDARDS

A.	ANSI/ICC A117.1 - American National Standard for Accessible and Usable Buildings and Facilities; International Code Council; 2003.

B.	ATBCB ADAAG - Americans with Disabilities Act Accessibility Guidelines; 2002.

1.04	SUBMITTALS

A.	See Section 01 3000 - Administrative Requirements, for submittal procedures.

B.	Product Data:  Manufacturer's printed product literature for each type of sign, indicating sign styles, font, foreground and background colors, locations, overall dimensions of each sign.

C.	Signage Schedule:  Provide information sufficient to completely define each sign for fabrication, including room name, other text to be applied, sign and letter sizes, fonts, and colors.

1.05	QUALITY ASSURANCE

A.	Manufacturer Qualifications:  Company specializing in manufacturing the products specified in this section with minimum three years of documented experience.

1.06	DELIVERY, STORAGE, AND HANDLING

A.	Package signs as required to prevent damage before installation.

B.	Package room and door signs in sequential order of installation, labeled by floor or building.

C.	Store tape adhesive at normal room temperature.

1.07	FIELD CONDITIONS

A.	Do not install tape adhesive when ambient temperature is lower than recommended by manufacturer.

B.	Maintain this minimum temperature during and after installation of signs.

PART 2  PRODUCTS

2.01	SIGNAGE APPLICATIONS

A.	Accessibility Compliance:  All signs are required to comply with ADAAG and ANSI/ICC A 117.1 and applicable building codes, unless otherwise indicated; in the event of conflicting requirements, comply with the most comprehensive and specific requirements.

B.	Room and Door Signs:  Provide a sign for every doorway.

1.	Sign Type:  Flat signs with engraved panel media as specified.

2.	Provide "tactile" signage, with letters raised minimum 1/32 inch and Grade II braille.

3.	Character Height:  1 inch.

4.	Sign Height:  9 inches, unless otherwise indicated.

5.	Rest Rooms:  Identify with pictograms, the names "Restroom", and braille.

2.02	SIGN TYPES

A.	Flat Signs:  Signage media without frame.

1.	Edges:  Square.

2.	Corners:  Rounded.

3.	Wall Mounting of One-Sided Signs:  Tape adhesive.

B.	Color and Font:  Unless otherwise indicated:

1.	Character Font:  Helvetica, Arial, or other sans serif font.

2.	Character Case:  Upper case only.

3.	Background Color:  To be selected by Architect from manufacturer’s complete range of standard and custom colors.

4.	Character Color:  Contrasting color.

2.03	TACTILE SIGNAGE MEDIA

A.	Engraved Panels:  Laminated colored plastic; engraved through face to expose core as background color:

1.	Total Thickness:  1/16 inch.

2.04	NON-TACTILE SIGNAGE MEDIA

A.	Silk Screened Plastic Panels:  Letters and graphics silk screened onto reverse side of plastic surface:

1.	Sign Color:  To be selected by Architect from manufacturer’s complete range of standard and custom colors.

2.	Total Thickness:  1/8 inch.

2.05	ACCESSORIES

A.	Concealed Screws:  Stainless steel, galvanized steel, chrome plated, or other non-corroding metal.

B.	Tape Adhesive:  Double sided tape, permanent adhesive.

PART 3  EXECUTION

3.01	INSTALLATION

A.	Install in accordance with manufacturer's instructions.

B.	Install neatly, with horizontal edges level.

C.	Locate signs where indicated:

1.	Room and Door Signs:  Locate on wall at latch side of door with centerline of sign at 60 inches above finished floor.

D.	Protect from damage until Substantial Completion; repair or replace damage items.

END OF SECTION 10 1400