SECTION 10 2113.19

PLASTIC TOILET COMPARTMENTS

PART 1  GENERAL

1.01	SECTION INCLUDES

A.	Solid plastic toilet compartments.

B.	Urinal screens.

1.02	RELATED REQUIREMENTS

A.	Section 10 2800 - Toilet, Bath, and Laundry Accessories.

1.03	SUBMITTALS

A.	See Section 01 3000 - Administrative Requirements, for submittal procedures.

B.	Shop Drawings:  Indicate partition plan, elevation views, dimensions, details of wall supports, door swings.

C.	Product Data:  Provide data on panel construction, hardware, and accessories.

D.	Samples:  Submit two samples of partition panels, four x four inch in size illustrating panel finish, color, and sheen.

E.	Manufacturer's Installation Instructions:  Indicate special procedures.

PART 2  PRODUCTS

2.01	MANUFACTURERS

A.	Plastic Toilet Compartments:

1.	Accurate Partitions Corporation; www.accuratepartitions.com.

2.	Ampco Products, Inc:  www.ampco.com.

3.	Global Partitions:  www.globalpartitions.com.

4.	Substitutions:  Section 01 6000 - Product Requirements.

2.02	COMPONENTS

A.	Toilet Compartments:  Solid molded phenolic through-color plastic panels, doors, and pilasters, floor-mounted unbraced.

B.	Door and Panel Dimensions:

1.	Thickness:  1 inch.

2.	Door Width:  24 inch.

3.	Door Width for Handicapped Use:  36 inch.

4.	Height:  58 inch.

5.	Thickness of Pilasters:  1 inch.

C.	Urinal Screens:  Wall mounted with two panel brackets, and floor-to-ceiling vertical upright consisting of pilaster anchored to floor and ceiling.

2.03	ACCESSORIES

A.	Pilaster Shoes:  Formed chromed steel with polished finish, 3 in high, concealing floor fastenings.

1.	Provide adjustment for floor variations with screw jack through steel saddles integral with pilaster.

B.	Pilaster Brackets:  Polished stainless steel.

C.	Wall Brackets:  Continuous type, polished stainless steel.

D.	Attachments, Screws, and Bolts:  Stainless steel, tamper proof type.

1.	For attaching panels and pilasters to brackets:  Through-bolts and nuts; tamper proof.

E.	Hardware:  Polished stainless steel:

1.	Pivot hinges, gravity type, adjustable for door close positioning; two per door.

2.	Door Latch:  Slide type with exterior emergency access feature.

3.	Door strike and keeper with rubber bumper; mounted on pilaster in alignment with door latch.

4.	Coat hook with rubber bumper; one per compartment, mounted on door.

5.	Provide door pull for out-swinging doors.

PART 3 EXECUTION

3.01	EXAMINATION

A.	Verify that field measurements are as indicated.

B.	Verify correct spacing of and between plumbing fixtures.

C.	Verify correct location of built-in framing, anchorage, and bracing.

3.02	INSTALLATION

A.	Install partitions secure, rigid, plumb, and level in accordance with manufacturer's instructions.

B.	Maintain 3/8 to 1/2 inch space between wall and panels and between wall and end pilasters.

C.	Attach panel brackets securely to walls using anchor devices.

D.	Attach panels and pilasters to brackets.  Locate head rail joints at pilaster center lines.

E.	Field touch-up of scratches or damaged finish will not be permitted.  Replace damaged or scratched materials with new materials.

3.03	TOLERANCES

A.	Maximum Variation from True Position:  1/4 inch.

B.	Maximum Variation from Plumb:  1/8 inch.

3.04	ADJUSTING

A.	Adjust and align hardware to uniform clearance at vertical edge of doors, not exceeding 3/16 inch.

B.	Adjust hinges to position doors in partial opening position when unlatched.  Return out-swinging doors to closed position.

C.	Adjust adjacent components for consistency of line or plane.

END OF SECTION 10 2113.19