Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 10 2800

TOILET, BATH, AND LAUNDRY ACCESSORIES

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Public-use washroom accessories.

          - Warm-air dryers.

          - Childcare accessories.

          - Custodial accessories.

For large industrial projects, campus-type projects with multiple buildings, or similar situations, Owner may have separate service contracts for furnishing various accessories and corresponding supplies.  If Owner will furnish accessories to Contractor for installation, insert requirements in first paragraph below.

      - ACTION SUBMITTALS

        - Product Data:  For each type of product indicated.  Include the following:

          - Construction details and dimensions.

          - Anchoring and mounting requirements, including requirements for cutouts in other work and substrate preparation.

          - Material and finish descriptions.

          - Features that will be included for Project.

          - Manufacturer's warranty.

If retaining first paragraph below, consider revising to limit sample requirements to specific items.

        - Product Schedule:  Indicating types, quantities, sizes, and installation locations by room of each accessory required.

          - Identify locations using room designations indicated.

Retain subparagraph below if product designations are indicated.  Coordinate with Part 2 articles that specify products.

          - Identify products using designations indicated.

      - INFORMATIONAL SUBMITTALS

        - Warranty:  Sample of special warranty.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data:  For toilet and bath accessories to include in maintenance manuals.

      - QUALITY ASSURANCE

        - Source Limitations:  For products listed together in the same Part 2 articles, obtain products from single source from single manufacturer.

Retain paragraph below if warm-air dryers are required.

        - Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

      - COORDINATION

        - Coordinate accessory locations with other work to prevent interference with clearances required for access by people with disabilities, and for proper installation, adjustment, operation, cleaning, and servicing of accessories.

        - Deliver inserts and anchoring devices set into concrete or masonry as required to prevent delaying the Work.

- PRODUCTS

      - MATERIALS

Retain this article if not naming manufacturers' products and where material descriptions will be used in part to determine the selection of products.

        - Stainless Steel:  ASTM A 666, Type 304, 0.031-inch (0.8-mm) minimum nominal thickness unless otherwise indicated.

        - Brass:  ASTM B 19, flat products; ASTM B 16/B 16M, rods, shapes, forgings, and flat products with finished edges; or ASTM B 30, castings.

        - Steel Sheet:  ASTM A 1008/A 1008M, Designation CS (cold rolled, commercial steel), 0.036-inch (0.9-mm) minimum nominal thickness.

        - Galvanized-Steel Sheet:  ASTM A 653/A 653M, with G60 (Z180) hot-dip zinc coating.

        - Galvanized-Steel Mounting Devices:  ASTM A 153/A 153M, hot-dip galvanized after fabrication.

        - Fasteners:  Screws, bolts, and other devices of same material as accessory unit and tamper-and-theft resistant where exposed, and of galvanized steel where concealed.

Retain first paragraph below for chrome finish.  Brass, steel, and cast-zinc alloy (zamac) are available with chrome finish.

        - Chrome Plating:  ASTM B 456, Service Condition Number SC 2 (moderate service).

Revise first paragraph below if tempered or safety glass is required; ASTM C 1503 does not cover tempered or safety glass mirrors.  Note that reflected images with tempered or safety glass may be distorted.

        - Mirrors:  ASTM C 1503, Mirror Glazing Quality, clear-glass mirrors, nominal 6.0 mm thick.

        - ABS Plastic:  Acrylonitrile-butadiene-styrene resin formulation.

Products offered by manufacturers vary.  Verify products offered by manufacturers in each article, and revise product characteristics accordingly.

      - PUBLIC-USE WASHROOM ACCESSORIES

        - Basis-of-Design Product:  Subject to compliance with requirements, provide product indicated on Drawings or comparable product by one of the following:

          - A & J Washroom Accessories, Inc.

          - American Specialties, Inc.

          - Bobrick Washroom Equipment, Inc.

          - Bradley Corporation.

Retain "Basis-of-Design Product" subparagraphs in remainder of this article if applicable.  Coordinate with specification method retained above.

Copy paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required.  Use these designations on Drawings to identify each product.

        - Toilet Tissue (Roll) Dispenser (TPD-1):

          - Basis-of-Design Product: Bobrick B-2888.

          - Description:  Roll-in-reserve dispenser with hinged front secured with tumbler lockset.

          - Mounting:  Surface mounted.

Controlled-delivery units cannot be used at accessible toilets.

          - Operation:  Noncontrol delivery with theft-resistant spindle.

          - Capacity:  Designed for 5-inch- (127-mm-) diameter tissue rolls.

          - Material and Finish:  Stainless steel, No. 4 finish (satin).

Combination toilet tissue dispenser units are available with sanitary-napkin disposal units, seat-cover dispensers, or both.

Coordinate requirements for soap dispensers with other Sections if a central soap storage tank is included.

        - Liquid-Soap Dispenser (SD):

          - Basis-of-Design Product: Bobrick B-830 “SureFlo” system.

          - Description:  Designed for dispensing soap in liquid or lotion form.

          - Mounting:  Deck mounted on vanity.

          - Capacity: 12 litres.

          - Materials: Stainless steel.

          - Lockset:  Tumbler type.

Retain last subparagraph above and subparagraph below if required.

          - Refill Indicator:  Window type.

        - Grab Bar (GB-1, GB-2, GB-3):

          - Basis-of-Design Product: Bobrick B- 6806.

          - Mounting:  Flanges with concealed fasteners.

          - Material:  Stainless steel, 0.05 inch (1.3 mm) thick.

            - Finish:  Smooth, No. 4 finish (satin) on ends and slip-resistant texture in grip area.

Retain first option in first subparagraph below for medium-duty applications, second option for heavy duty.

          - Outside Diameter:  1-1/2 inches (38 mm).

          - Configuration and Length:  As indicated on Drawings.

        - Sanitary-Napkin Disposal Unit (SND):

          - Basis-of-Design Product: Bobrick B-254 “ClassicSeries”.

          - Mounting:  Surface mounted.

          - Door or Cover:  Self-closing, disposal-opening cover and hinged face panel with tumbler lockset.

          - Receptacle:  Removable.

          - Material and Finish:  Stainless steel, No. 4 finish (satin).

        - Mirror Unit (MIR):

          - Basis-of-Design Product: American Specialties #0620; Type 304 stainless steel mirror with #8 finish.

Tilted and adjustable tilting mirrors are for use by people with disabilities.  Adjustable tilting mirrors are prone to vandalism.  In lieu of tilted mirrors, standard flat mirrors can be mounted at heights to accommodate users in wheelchairs.

          - Frame:  Stainless-steel channel.

            - Corners:  Manufacturer's standard.

Retain first subparagraph below if required.

          - Hangers:  Produce rigid, tamper- and theft-resistant installation, using method indicated below.

            - One-piece, galvanized-steel, wall-hanger device with spring-action locking mechanism to hold mirror unit in position with no exposed screws or bolts.

Delete last subparagraph above or first subparagraph below, or retain both and indicate as Contractor's option.  Delete below if tilting mirrors are required.

            - Wall bracket of galvanized steel, equipped with concealed locking devices requiring a special tool to remove.

          - Size:  As indicated on Drawings.

      - WARM-AIR DRYERS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two paragraphs and list of manufacturers below.  See Section 016000 "Product Requirements."

        - Basis-of-Design Product:  Subject to compliance with requirements, provide product indicated on Drawings or comparable product by one of the following:

          - A & J Washroom Accessories, Inc.

          - American Dryer, Inc.

          - American Specialties, Inc.

          - Bobrick Washroom Equipment, Inc.

          - Bradley Corporation.

          - Excel Dryer Corporation.

          - GAMCO Specialty Accessories; a division of Bobrick Washroom Equipment, Inc.

          - Tubular Specialties Manufacturing, Inc.

          - World Dryer Corporation.

Copy paragraph below and re-edit for each product.

Insert number to complete drawing designation for each product required.  Use these designations on Drawings to identify each product.

        - Warm-Air Dryer (EHD):

          - Basis-of-Design Product: Bobrick B-7128 115V TrimLine Series, ADA Hand Dryer.

          - Mounting:  Surface mounted.

          - Operation:  Electronic-sensor activated with timed power cut-off switch.

            - Operation Time:  Manufacturer’s standard (40 seconds minimum).

          - Cover Material and Finish:  Stainless steel, No. 4 finish (satin).

          - Electrical Requirements 115 volt AC, 15 amp, 1725 watts, 50/60 hrtz .

      - CHILDCARE ACCESSORIES

        - Basis-of-Design Product:  Subject to compliance with requirements, provide product indicated on Drawings or comparable product by one of the following:

          - American Specialties, Inc.

          - Brocar Products, Inc.

          - Diaper Deck & Company, Inc.

          - GAMCO Specialty Accessories; a division of Bobrick Washroom Equipment, Inc.

          - Koala Kare Products; a division of Bobrick Washroom Equipment, Inc.

          - SSC, Inc.

          - Tubular Specialties Manufacturing, Inc.

Retain "Basis-of-Design Product" subparagraphs in remainder of this article if applicable.  Coordinate with specification method retained above.

Copy paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required.  Use these designations on Drawings to identify each product.

        - Diaper-Changing Station (BCS):

          - Basis-of-Design Product: Bobrick, Koala Kare KB110-SSWM.

          - Description:  Horizontal unit that opens by folding down from stored position and with child-protection strap.

            - Engineered to support a minimum of 250-lb (113-kg) static load when opened.

          - Mounting:  Surface mounted, with unit projecting not more than 4 inches (100 mm) from wall when closed.

          - Operation:  By pneumatic shock-absorbing mechanism.

          - Material and Finish:  Stainless steel, No. 4 finish (satin), with replaceable insulated polystyrene tray liner and rounded plastic corners.

          - Liner Dispenser:  Built in.

      - CUSTODIAL ACCESSORIES

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two paragraphs and list of manufacturers below.  See Section 016000 "Product Requirements."

        - Basis-of-Design Product:  Subject to compliance with requirements, provide product indicated on Drawings or comparable product by one of the following:

          - A & J Washroom Accessories, Inc.

          - American Specialties, Inc.

          - Bobrick Washroom Equipment, Inc.

          - Bradley Corporation.

          - GAMCO Specialty Accessories; a division of Bobrick Washroom Equipment, Inc.

          - Tubular Specialties Manufacturing, Inc.

Retain "Basis-of-Design Product" subparagraphs in remainder of this article if applicable.  Coordinate with specification method retained above.

Copy paragraphs below and re-edit for each product.

Insert number to complete drawing designation for each product required.  Use these designations on Drawings to identify each product.

        - Utility Shelf:

          - Basis-of-Design Product: Bobrick B-295 x 24.

          - Description:  With exposed edges turned down not less than 1/2 inch (13 mm) and supported by two triangular brackets welded to shelf underside.

          - Material and Finish:  Not less than nominal 0.05-inch- (1.3-mm-) thick stainless steel, No. 4 finish (satin).

        - Mop and Broom Holder:

          - Basis-of-Design Product: Bobrick B-223 x 24.

          - Mop/Broom Holders: Three, spring-loaded, rubber hat, cam type.

          - Material and Finish:  Stainless steel, No. 4 finish (satin).

      - FABRICATION

        - General:  Fabricate units with tight seams and joints, and exposed edges rolled.  Hang doors and access panels with full-length, continuous hinges.  Equip units for concealed anchorage and with corrosion-resistant backing plates.

        - Keys:  Provide universal keys for internal access to accessories for servicing and resupplying.  Provide minimum of six keys to Owner's representative.

- EXECUTION

      - INSTALLATION

        - Install accessories according to manufacturers' written instructions, using fasteners appropriate to substrate indicated and recommended by unit manufacturer.  Install units level, plumb, and firmly anchored in locations and at heights indicated.

        - Grab Bars:  Install to withstand a downward load of at least 250 lbf (1112 N), when tested according to ASTM F 446.

      - ADJUSTING AND CLEANING

        - Adjust accessories for unencumbered, smooth operation.  Replace damaged or defective items.

        - Remove temporary labels and protective coatings.

        - Clean and polish exposed surfaces according to manufacturer's written recommendations.

END OF SECTION 10 2800