SECTION 12 3661

SIMULATED STONE COUNTERTOPS

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Solid-surface-material countertops and backsplashes.

          - Quartz agglomerate countertops and backsplashes.

        - Related Sections:

          - See plumbing drawings for related information.

      - ACTION SUBMITTALS

        - Product Data:  For countertop materials.

        - Shop Drawings: For countertops. Show materials, finishes, edge and backsplash profiles, methods of joining, and cutouts for plumbing fixtures.

        - Samples for Initial Selection:  For each type of material exposed to view.

        - Samples for Verification:  For the following products:

          - Countertop material, 6 inches (150 mm) square.

      - PROJECT CONDITIONS

        - Field Measurements: Verify dimensions of countertops by field measurements before countertop fabrication is complete.

      - COORDINATION

        - Coordinate locations of utilities that will penetrate countertops or backsplashes.

- PRODUCTS

      - SOLID-SURFACE-MATERIAL COUNTERTOPS (SS-1)

        - Countertops:  1/4-inch- (6.4-mm-) thick, solid surface material laminated to 3/4-inch- (19-mm-) thick particleboard with exposed edges built up with 3/4-inch- (19-mm-) thick, solid surface material unless otherwise detailed on drawings.

        - Backsplashes:  3/4-inch- (19-mm-) thick, solid surface material.

        - Fabrication:  Fabricate tops in one piece with shop-applied edges and backsplashes unless otherwise indicated.  Comply with solid-surface-material manufacturer's written instructions for adhesives, sealers, fabrication, and finishing.

      - QUARTZ AGGLOMERATE COUNTERTOPS (SS-2)

        - Countertops:  3/4-inch- (19-mm-) thick, quartz agglomerate with front edge built up with same material.

        - Backsplashes:  3/4-inch- (19-mm-) thick, quartz agglomerate.

        - Fabrication: Fabricate tops in one piece with shop-applied edges and backsplashes unless otherwise indicated. Comply with quartz agglomerate manufacturer's written instructions for adhesives, sealers, fabrication, and finishing.

      - COUNTERTOP MATERIALS

        - Plywood: Exterior softwood plywood complying with DOC PS 1, Grade C-C Plugged, touch sanded.

        - Adhesives:  Adhesives shall not contain urea formaldehyde.

        - Solid Surface Material: Homogeneous solid sheets of filled plastic resin complying with ANSI SS1.

          - Manufacturers: Subject to compliance with requirements, available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

            - Avonite Surfaces.

            - Formica Corporation.

            - Wilsonart International.

          - Type: Provide Standard Type or Veneer Type made from material complying with requirements for Standard Type, as indicated unless Special Purpose Type is indicated.

          - Colors and Patterns:  As selected by Architect from manufacturer's full range.

        - Quartz Agglomerate:  Solid sheets consisting of quartz aggregates bound together with a matrix of filled plastic resin and complying with the "Physical Characteristics of Materials" Article of ANSI SS1.

          - Manufacturers: Subject to compliance with requirements, available manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

            - Cambria.

            - Cosentino USA.

          - Colors and Patterns:  As selected by Architect from manufacturer's full range.

- EXECUTION

      - INSTALLATION

        - Install countertops level to a tolerance of 1/8 inch in 8 feet

        - Fasten countertops by screwing through corner blocks of base units into underside of countertop.  Pre-drill holes for screws as recommended by manufacturer.  Align adjacent surfaces and, using adhesive in color to match countertop, form seams to comply with manufacturer's written instructions.  Carefully dress joints smooth, remove surface scratches, and clean entire surface.

          - Seal edges of cutouts in particleboard subtops by saturating with varnish.

END OF SECTION 12 3661