Page 1

REQUEST FOR BID        RETURN BY OPENING TIME TO:

Date:02/24/11          Purchasing Division

Bid 11-07DS            RM 100 City Hall

411 West 1st Street

Duluth, MN 55802

Solar Powered Trash/Recycling

Kiosks

Buyer: Dennis Sears

Phone: 218-730-5003

Fax:   218-730-5922

BID OPENING, RM 100 AT 2:00 PM ON Tuesday, March 15, 2011

Note: All bids must be written, signed, and transmitted in a sealed envelope, plainly marked with the bid number, subject matter, and opening date. The City of Duluth reserves the right to split award where there is substantial savings to the city, waive informalities and to reject any and all bids. Bidder should state in proposal if bid is based on acceptance of total order. Sales tax is not to be included in the unit price. Bidder to state freight charges if, proposal is F.O.B. shipping point, freight not allowed. Low bid will not be the only consideration for award of bid. All pages must be signed or initialed by authorized bidder’s representative as indicated at the bottom of the page(s) of the request for bid forms.

RETURN BID IN DUPLICATE WITH DUPLICATE DESCRIPTIVE LITERATURE

BID RESULTS WILL BE POSTED TO THE WEB SITE SHORTLY AFTER BID OPENING

City bid information on website: www.duluthmn.gov/purchasing/bid_information.cfm

Designated F.O.B. Point

Tax: Federal Excise Tax Exemption

Account No. 41-74-0056 K

----------------------------------------------------------------------

Item   Qty   U/OM          Description                 Unit     Total

No.                                                   Price    Price

----------------------------------------------------------------------

01   10     ea   Two (2) unit solar kiosks-single    ______   ______

stream recycling unit + trash

compactor per attached description

and specifications.

Vendor E-mail Address_________________________ Freight Charges    N/A

Name____________________                     Total Bid Price ________

Addr____________________               (To include any additional pages)

____________________

____________________                     Payment Terms ___________

By:_______________________                   F.O.B. Point  N/A

(print title)

__________________________                   Delivery Date  N/A

(signature)    (tele#)

Pg 2

-----------------------------------------------------------------------

Each kiosk will include a solar trash compactor and a single stream recycler.

Compactor

Compactor will have a minimum capacity of 30 gals compacted trash.

The handle height on the compactor shall be ADA compliant.

The compacting force shall be a minimum of approximately 1250 lb.

The electronics on the compactor is a 12 volt DC system with a 30       watt PV Module protected by a polycarbonate bubble or equivalent        system.

Front of compactor shall state “Non-Recyclable Trash Only” at the       top and have the City of Duluth logo (supplied) at the bottom with

the words “making a difference for tomorrow-today underneath.

Recycling Unit

The recycling unit with have a minimum capacity of 50 gals.

The top of the recycling unit with details of what is to be/can be       Recycled: ”example-Recyclables-plastic, cans, glass, tin,

newsprint” The recycling symbol will be 5 plus inches in diameter       and located on the access door.

Unit

Please provide a color chart of available colors along with

complete specifications of kiosk being bid.

Each unit will have 2 ad panels (1 on each side) installed.

There will be one “CLEAN” wireless notification with the units.

All costs associated with the kiosks shall be included in the           cost.

Please attach a breakout of costs and carry total cost to bid           cover page.

___________________

(initials)